## VERSA-KING 3” SPECIFICATIONS

# PART 1 – GENERAL

1.1 Provide all labor, equipment and materials to furnish prefabricated Versa-King 3” (insert here) <modular wall system> or <modular room> or <modular enclosure>, as indicated on drawing.

# 1.2 RELATED SECTIONS

Installation

Electrical

Fire Suppression (If Applicable)

Mechanical / HVAC

Plumbing (If Applicable)

# 1.3 QUALITY ASSURANCE

Modular system shall be the product of a manufacturer with a minimum of 20 years documented experience in the design and fabrication of modular wall systems, rooms and enclosures.

# 1.4 REFERENCES

Upon request, the manufacturer shall provide the locations and points of contact for five (5) similar installations with comparable application criteria that have been in service for a minimum of five (5) years.  This will allow the architect / owner to evaluate the quality of materials and workmanship.

# 1.5 SUBMITTALS

Upon award of an order, manufacturer shall prepare and submit copies of shop drawings as required.  Drawings shall include elevations, section details and floor plan. Samples of panel finishes shall be submitted for owner’s approval.

# PART 2 – PRODUCTS

# 2.1 ACCEPTABLE MANUFACTURERS

A.  The approved manufacturer is Porta-King Building Systems, 4133 Shoreline Drive, Earth City, MO

63045 / Phone: 1-800-284-5346 / Fax: 1-314-291-2857 / E-mail: info@portaking.com

B.  Modular system(s) by manufacturers other that those pre-approved shall submit sufficient product

data for review prior to granting approved status.  As a minimum: Design drawings and / or

engineering calculations, applicable code compliance, product catalog(s) and samples of wall panel

construction.

2.2 SYSTEM COMPONENTS

A. Versa-King modular system shall be constructed from pre-finished materials supplied by Porta-King

Building Systems consisting of the following specific components:

B. Wall Panels:  Laminated design consisting of solid core with a pre-finished interior and exterior

surface.

- Wall panels are to be nominal 3” thick by 48” wide by (insert height).

Standard panel height is 8’-0” up to a maximum height of 24’-0”.

- Core: Expanded polystyrene ASTM E84 Class A fire-resistant, 1 pound

PSF density.

- Panel finish: Interior finish (insert finish), exterior finish (insert finish)

a. <White> <Beige> <Blue> <Gray> <Pecan wood grain> 1/8” vinyl

covered hardboard.

b. <White> <Beige> <Blue> <Almond> <Gray> <Pecan wood grain> ½”

vinyl-covered drywall with Class A fire rating in accordance with

ASTM E84.

c. <Beige> <White> 24 gauge painted steel.

d. <White> <Gray> <Almond> fiberglass reinforced plastic (FRP).

C. Structural Framework: Extruded aluminum alloy 6005T6 with a <clear

satin anodized 204R1> or <beige painted> finished surface.

1.    Standard Panel Connecting Post: Consist of a 3-piece “non-progressive” design which serves

as the connecting device for holding wall panels together.  These posts also serve as a vertical

chase to conceal electrical and data service.  A full height snap-on cover plate is included to

allow for easy access during initial installation and to enable future modification without

disassembly of wall system.  Standard connecting posts are also load-bearing and can be used

to create load-bearing and two-story modular buildings.

2.   Corner Post: Consist of a 2-piece “non-progressive” design which serves as both a structural

member and as the connecting device for wall panels at 90 degree angles.

3.   Non-progressive design: Standard panel connecting post and corner post enable wall panels to

be removed or replaced without disassembly of adjacent wall panels.

4.    Wall Starts:  Consist of a 12 gauge steel U-shaped channel to fit flush against existing

structure or where two modular panels meet on a perpendicular line.  The U-shaped channel

is sized to enable standard panel connectors or corner post to positively connect while

enabling panel installation on the adjacent side.

5.  Ceiling Cap: Extruded aluminum alloy 6005T6 provided in pre-cut, pre-mitered lengths to fit

snugly around the top of all perimeter wall sections eliminating on-site fabrication.  An

integral vertical fascia is included to create a neat, finished appearance.

6.  Partition Cap: Extruded U-shaped aluminum alloy 6005T6 provided in pre-cut, pre-mitered

lengths to fit snugly around the top of all partition wall sections eliminating on-site

fabrication.

7.  Base Plate: Extruded U-shaped aluminum alloy 6005T6 provided in pre-cut, pre-mitered

lengths to serve as a leveling surface and guide for installation of wall panels and connecting

posts.

2.3   DOOR ASSEMBLIES:  Full flush doors are shipped pre-hung in frame.  Includes lock set shipped

separately.  Doors include an integral threshold and stationary door sweep.

- Door Leaf: Provide (insert here)

- <3068> or <3070> HD single swing door is constructed of a heavy-duty structural anodized aluminum alloy framework.  Store-front type construction utilizing tubular extruded components, mechanically fastened together to form a rigid frame.

- <3068> or <3070> single swing door, hollow metal steel door consisting of 18 gauge sheet steel.

- <3068> or <3070> single swing door is hollow-core door with pre-finished, embossed hardboard construction in light oak wood grain finish.

- <3068> or <3070> single swing door is solid-core door with pre-finished, embossed hardboard construction in light oak wood grain finish.

- Door Frame:  Extruded aluminum alloy 6005T6 with a <clear, satin anodized 204R1> or <beige painted> finish with integral stop, threshold and full perimeter rubber bulb gasket.

- Door Hardware: Provide as specified in section 08710 – Door Hardware.

1.  Hinges: Fully mortised, plain bearing 4 ½” x 4” butt hinge with a prime coat aluminum

finish.

2.   Locksets:  Industrial grade, <cylindrical> or <lever> as specified by architect.

3.  Door Sweep: Aluminum extrusion with black vinyl sweep, surface mounted

to the bottom of the door leaf.

D.   Vision Lite:  Provide factory installed 20” wide by 20” high vision lite with <1/8” tempered safety

glass> or <3/16” tempered safety glass> or <5/8” insulated, dual paned, tempered safety glass> or

<1/4” laminated safety glass> or <3/16” Plexiglas> or <3/16” acrylic>.

Note, when specifying heavy duty swing door the vision lite is 29” wide by 39” high.

2.4   WINDOW ASSEMBLIES: Consist of a three-piece system which includes the window frame

and wall panel sections to fit above and below to create a full height panel assembly.

A.   Frame:  <Fixed> or <Horizontal Sliding> type, constructed of extruded aluminum

alloy 6005T6 with a <clear satin anodized 204R1> or <beige painted> finish. Window frames will

be preassembled, including specified factory glazing.  Frames will positively lock into adjoining

connecting posts. Fixed window frames will include removable stops to enable easy replacement

of glazing.  Header and sill panels will be installed in conjunction with frame to create a complete

full-height wall panel.

B.  Glazing:  <3/16” tempered safety glass> or <5/8” insulated, duel paned, tempered

safety glass> or <1/4” laminated safety glass> or <3/16” Plexiglas> or <3/16”

acrylic>.  Factory installed in frame with full perimeter rubber bulb gasket.

2.5   ROOF SYSTEMS

A.  Steel Deck: Corrugated B-deck serves as roof cover and span support for acoustical

grid ceiling system & lights. Utilize <22 gauge for clear spans up to 12’ wide> or <18

gauge for clean spans from 12’ to16’ wide>.

B.   For spans of considerable length, structural I-beams may be required to reduce clear span

conditions.  I-beams will be mechanically fastened to the top of panel connectors on centerlines as

required for different span conditions.

C.   Acoustical Ceiling System: Suspended metal grid with lay-in acoustical tile.

1.  Ceiling grid: Double webbed, commercial grade, non-rated, galvanized steel with white

baked enamel paint finish.

2.  Acoustical ceiling tile: 2’x4’ / 5/8” non-directional fissured and perforated wet-felted

lay-in panels and tile.  Flame Spread Classification Class A.

D.   Panelized Ceiling System:  Consists of a laminated panel nominally 3” thick by 46”

wide with a 26 gauge embossed steel laminated to both sides of a solid polystyrene

core.  Panels are placed on top of wall panels and assembled via a tongue and

groove connection.  Panels are not designed to support storage loading.

2.6   ELECTRICAL COMPONENTS

A.  Electrical Outlets: Provide as specified on drawing.  Outlets are to be installed inside the chase of

the panel connecting post.

1.  Duplex outlets:  115volt, 3 wire, 20 amp capacity installed 15” above finished floor.

2.  Single Outlets:  230volt, 3 wire, 15 amp capacity installed 15” above finished floor.

B.   Data / Telecommunications Jacks:

1.  Single port for <telecommunications> or <data> service provided by others.  To be

installed 15” above the finished floor.

2.  Dual port for <telecommunications> or <data> service provided by others.  To be

installed 15” above the finished floor.

C. Lighting:  Provide as indicated on drawing or design to provide (insert here) foot-candles

1.  Fixtures:

a. Provide 120 volt, 2’x4’ grid lay-in, 160 watt, with four 40 watt tubes (supplied by

others).

b. Provide 120 volt, 2’x4’ surface mounted, 160 watt with four 40 watt tubes (supplied

by others).

2.  Light Switch: Provide <single> or <double> or <3-way> as specified on drawing.  To be

installed 40 3/4” above the finished floor.

D.  Load Center:  Main lug, single phase, 100 amp capacity, (      ) circuits with 20 amp

circuit breakers.

2.7   AIR CONDITIONING:

A.  Through-wall <A.C.> or <H.V.A.C.> unit(s), to be installed in factory framed wall panel

cutout as indicated on drawing.

1. Air Conditioner (A.C.):

a. <115 volt, 6,800 BTU> or <115 volt 8,000 BTU> or <230 volt, 11,600 BTU>

2. Combination H.V.A.C.:

a. <115 volt, 6,000 Cool / 4,100 Heat BTU> or <230 volt, 9,900 Cool / 11,600 Heat>

or <230 volt 11,600 Cool / 11,600 Heat BTU> or <230 volt, 17,800 Cool / 15,800

Heat BTU>

3. Framed Cutout:  Provided factory installed frame, constructed of extruded aluminum

alloy 6005T6 with a <clear satin anodized 204R1> or <beige painted> finish.

C.  Central heating, ventilating and air conditioning system in accordance to specifications outlined

in Division 15.

# PART 3 – EXECUTION

# 3.1   PREPARATATION

A. Coordinate on-site delivery and off-loading of modular system to ensure a secure staging area with

sufficient floor space to store modular construction material.  Confirm access way is

free and clear of obstructions to enable delivery of modular system to the construction site.

Confirm lighting and electrical provisions are adequate and easily accessible.

3.2   ERECTION

A. Install modular system on a flat and level surface in accordance with the manufacturer’s

installation instructions and drawings.  For technical assistance call 1-800-284-5346.